FACE RECOGNITION USING RASPBERRY PI 5 – PROJECT REPORT

By Aman Vajpayee

1. Introduction

The objective of this project was to build a complete face-recognition system on a Raspberry Pi 5 that can detect and recognize trained faces in real time, process camera frames efficiently using OpenCV, and train a custom model using images collected through the Pi. Unlike the original reference which used the Pi Camera Module, this project uses an LG USB Webcam with OpenCV’s VideoCapture.

2. Hardware Used

• Raspberry Pi 5 (8 GB)

• LG USB Webcam (USB connection)

• MicroSD card with Raspberry Pi OS (Bookworm)

3. Setting Up the Development Environment

3.1 Virtual Environments

Raspberry Pi OS Bookworm requires Python virtual environments for isolating project dependencies.

3.2 Creating the Virtual Environment

python3 -m venv --system-site-packages face_rec

3.3 Activating the Environment

source face_rec/bin/activate

4. Installing Required Libraries

sudo apt update

sudo apt full-upgrade

pip install opencv-python

pip install imutils

sudo apt install cmake

pip install face-recognition

5. Configuring Thonny to Use the Virtual Environment

Interpreter set to ~/face_rec/bin/python3.

6. Dataset Preparation and Image Capture

Dataset folder structure created inside project directory.

Using LG webcam with:

cam = cv2.VideoCapture(0)

Images captured with spacebar, script exits with Q. PERSON_NAME variable defines folder name.

7. Model Training

model_training.py processes dataset images, extracts face encodings, and generates encodings.pickle.

8. Real-Time Face Recognition

Replaced PiCam code with:

cam = cv2.VideoCapture(0)

System performs detection, scaling, face recognition, and bounding-box labeling.

9. Performance Tuning

Resolution set via:

cam.set(cv2.CAP_PROP_FRAME_WIDTH, 1920)

cam.set(cv2.CAP_PROP_FRAME_HEIGHT, 1080)

Processed resolution controlled with:

cv_scaler = 4

10. Results

The system successfully recognizes trained faces, operates smoothly with tuned scaling values, and performs accurately under varied lighting and minor movement.