# Project Report: Adding Multi‑User Face Recognition, MQTT Motor Control, and Email Alerts

This document explains all changes, upgrades, and integrations added to the original face‑recognition project, including:

- Multi‑user recognition (Aman + Jake Paul)

- Different motor actions per user

- MQTT‑based remote servo rotation through ESP32

- Gmail alert notifications with image attachment

- Camera configuration fixes (LG USB camera)

## 1. Addition of a Second User ("Jake Paul")

### What was done

- New face images for Jake Paul were collected using the existing image_capture.py script.

- These images were saved under: dataset/jake paul/

- The model_training.py script was re‑run to regenerate encodings.pickle.

### Result

The system can now recognize:

- "Aman Vajpayee"

- "jake paul"

Each name now triggers a different hardware behavior.

## 2. Per‑User Actions (Different Motor Movements)

### Why I added this

The requirement was:

- When Aman's face is detected → rotate motor 180°

- When Jake Paul's face is detected → rotate motor 360°

### What was added

A dictionary was introduced in the Python script:

authorized_actions = {

"Aman Vajpayee": "servo180",

"jake paul": "spin360"

}

### How it works now

- When a face is matched, the script checks the action mapped to that name.

- It then publishes a different MQTT command:

  - {"angle": 180} → for Aman

  - {"spin": true} → for Jake Paul

## 3. MQTT Integration to Control Servo Motor on ESP32

### Why MQTT was chosen

HTTP requests were failing on your mobile hotspot due to:

- Packet drops

- No route to host

- Hotspot's NAT restrictions

MQTT is lightweight, stable, and works over the public broker test.mosquitto.org.

### New components added

- Added MQTT client initialization

- Added safe reconnect logic

- Added wait_for_publish() to ensure messages are delivered

- Added two different publish functions:

  - trigger_motor_mqtt(angle)

  - trigger_motor_spin_command()

### Final behavior

- When the Pi detects a face → sends MQTT message to topic:

face_control/esp32/rotate

- ESP32 receives data and rotates the servo accordingly.

## 4. ESP32 Code Modifications

### Why ESP code was rewritten

We needed:

- MQTT subscription handling

- Parsing JSON payloads

- Secure token check

- Two different actions (angle & spin)

- LED blink support (future use)

### Key changes

- ESP subscribes to the topic at boot.

- ESP parses JSON values:

  - angle → rotate servo

  - spin → do two 180° sweeps (360° simulation)

- Added serial logs for full debugging

### Pin assignment

Servo Signal → GPIO 13

Servo VCC → 5V external power

Servo GND → SAME ground as ESP32

## 5. Gmail Alert System Integration

### What we added

Whenever an authorized face is detected:

- A snapshot is taken

- Encoded as JPEG

- Attached in email

- Sent via Python's SMTP_SSL library

### Requirements configured

GMAIL_USER = "amanvajpayee21@gmail.com"

GMAIL_PASS = "your app password"

(Google App Password was used, not real password.)

### Result

A formal alert email is received every time a known face appears.

## 6. Camera Fixes for LG Smart Cam

Originally, the camera produced:

select() timeout

This meant the camera was detected but was not streaming frames.

### Fixes applied

- Verified /dev/video0 correctly corresponds to LG Smart Cam.

- Tested video capture manually.

- Replugged USB device.

- Changed USB port.

- Rebooted the Pi.

After these actions, the camera delivered frames successfully.

## 7. Final Workflow Summary

- Webcam captures frames

- Face recognized using HOG model

- System checks which user appeared

- Depending on the user:

  - MQTT "angle" command is sent OR

  - MQTT "spin" command is sent

- ESP32 receives the command

- Servo rotates

- Pi sends Gmail alert with proof image

Everything now works reliably.

## 8. Files Modified in the Project

### Modified

- facial_recognition_hardware.py (major changes)

  - Added multi-user support

  - Added MQTT logic

  - Added email alerts

  - Added improved cooldown logic

  - Added per-user actions

### Re-generated

- encodings.pickle

### ESP32 Code

- Fully replaced with MQTT-based servo control

## 9. Final Notes

This upgraded system is now capable of:

- Multi-user recognition

- Remote servo actuation over MQTT

- Independent behavior per user

- Automatic email alerts

- Robust webcam streaming

This is a complete, production-ready IoT + AI system combining:
 Computer Vision + Edge AI + MQTT + Embedded Systems + Cloud Email Alerts.