# Face Recognition → Remote ESP32 Servo + Gmail Alerts

Author: Aman Vajpayee
 System: Raspberry Pi 5 (LG USB Webcam) + ESP32 (SG90 servo)
 Date: (use current date)

## 1. Objective

Build a working pipeline where:

- A Raspberry Pi runs face recognition (using an LG USB webcam) and recognizes trained faces using an encodings model (encodings.pickle).

- When an authorized face is detected:

  - The Pi publishes a command via MQTT to an ESP32 which controls a SG90 servo (rotate to 180° then return to 0°).

  - The Pi sends an email (Gmail) with a snapshot attachment to a configured recipient.

- The system uses a 30-second cooldown per identity to avoid repeats.

- The Pi runs everything in a Python virtual environment (venv).

## 2. Files (what was created/edited)

All paths below refer to my Raspberry Pi filesystem.

Primary file (final working script)

- /home/aman/Desktop/Face Recognition/facial_recognition_hardware.py

  - This is the corrected, final Python script that:

    - Opens the LG webcam,

    - Loads encodings.pickle,

    - Detects faces and names,

    - Publishes MQTT messages to the ESP32,

    - Sends Gmail alerts with snapshot attachments,

    - Uses cooldown and threading so detection is not blocked by IO.

Backup of corrupted file

- /home/aman/Desktop/Face Recognition/facial_recognition_hardware.py.bak

  - This is an automatic backup of whatever was in the file before it was overwritten.

Encodings (model used by the script)

- /home/aman/Desktop/Face Recognition/encodings.pickle

  - Generated earlier using model_training.py. This file contains the facial encodings and names arrays.

Other project files (original/unchanged)

- image_capture.py — used to capture images into the dataset/ (should use cv2.VideoCapture(0) for your LG webcam).

- model_training.py — trains encodings and writes encodings.pickle.

- facial_recognition.py — original recognition script (left unchanged except if you asked me to patch it).

## 3. Software components and libraries

On the Pi (Python venv face_rec)

- Python 3 (system)

- Virtual environment directory: ~/face_rec

- Important Python packages installed inside venv:

  - opencv-python

  - face-recognition (dlib based facial encodings)

  - imutils

  - paho-mqtt (MQTT client)

  - (built-in) smtplib and email for SMTP

On the ESP32 (Arduino core)

- Libraries required in Arduino IDE:

  - PubSubClient (by Nick O’Leary) — MQTT client

  - ArduinoJson — for JSON parsing

  - ESP32Servo — servo control

- Code uploaded to ESP32 is an MQTT subscriber that:

  - Connects to Wi-Fi (SSID: Red, password: 12345678),

  - Connects to test.mosquitto.org and subscribes to face_control/esp32/rotate,

  - On message { "token":"aman_face_access", "angle":180 }, rotates SG90 to 180°, waits 0.8s, then returns to 0°,

  - Logs received messages on the Serial Monitor.

## 4. Configuration used in development (explicit values)

Network

- Phone hotspot SSID: Red

- Hotspot password: 12345678
 (We used MQTT because many phone hotspots block direct client-to-client traffic.)

MQTT

- Broker used (public test): test.mosquitto.org (port 1883)

- Topic: face_control/esp32/rotate

- Shared token: aman_face_access

ESP32

- Static IP initially attempted, later changed to DHCP for hotspot compatibility.

- Final approach: MQTT subscriber on topic above.

- Servo pin used: GPIO 13 .

Gmail (email sending from Pi)

- Gmail user: amanvajpayee21@gmail.com

- App password (I created a 16-character App Password; it was used in the test script)

- Sent using smtp.gmail.com with SSL (port 465)

Cooldown

- COOLDOWN = 30 seconds per recognized name

Files

- Script path: /home/aman/Desktop/Face Recognition/facial_recognition_hardware.py

- Encodings path: /home/aman/Desktop/Face Recognition/encodings.pickle

## 5. Full run / install steps (what you must run to reproduce)

On Raspberry Pi — one-time installation

- Update system:

sudo apt update
sudo apt full-upgrade -y

- Create virtual environment (if not already):

python3 -m venv --system-site-packages ~/face_rec
source ~/face_rec/bin/activate

- Install required Python packages in venv:

pip install --upgrade pip
pip install opencv-python face-recognition imutils paho-mqtt

Note: face-recognition installs dlib which can take time and may need build-essential packages. I already had it installed.

Prepare dataset & train encodings

- Use image_capture.py (modified to cv2.VideoCapture(0)) to capture photos: set PERSON_NAME and press space to capture; press q to quit. Images will be saved under dataset/PERSON_NAME/.

- Run model_training.py to create encodings.pickle in your working folder.

ESP32 — upload the MQTT sketch

- Install PubSubClient, ArduinoJson, ESP32Servo via Arduino Library Manager.

- Upload the MQTT subscriber sketch to ESP32 (configure ssid password, mqtt_server, mqtt_topic same as above).

- Open Serial Monitor at 115200 to see logs and confirm subscription.

Run the Pi script

- Activate venv:

source ~/face_rec/bin/activate

- From the script directory or with absolute path run:

python3 "/home/aman/Desktop/Face Recognition/facial_recognition_hardware.py"

- Watch the outputs in the Pi terminal and check ESP32 Serial Monitor for MQTT messages and servo movement.

## 6. What I changed in my project (explicitly)

- Used VideoCapture instead of picamera cv2.VideoCapture(0) in scripts that capture images and in the recognition script so the LG USB webcam is used.

- Wrote a new facial_recognition_hardware.py:

  - Uses face_recognition library to find face locations and match encodings.

  - Uses paho-mqtt to publish rotation commands to MQTT broker.

  - Uses threaded send_email_alert() so sending email does not block detection.

  - Implements COOLDOWN per name to prevent repeats.

  - Includes graceful shutdown and cleanup.

- Provided and tested ESP32 code:

  - An HTTP server initial approach was switched to MQTT because phone hotspot blocked client-to-client traffic.

  - Final: MQTT subscriber sketch that receives rotate commands and rotates servo.

## 7. How it works — sequence diagram (textual)

- camera -> Pi: continuous frames captured (cv2.VideoCapture(0)).

- Pi: convert frame to RGB, detect face locations, compute encodings.

- Pi: compare encodings with encodings.pickle. If a name is recognized (not "Unknown"):

  - Check cooldown; if OK:

    - Spawn background thread: trigger_motor_mqtt(angle=180) ⇒ publishes {"token":"aman_face_access","angle":180} to test.mosquitto.org topic face_control/esp32/rotate.

    - Spawn background thread: send_email_alert(name, frame_copy) ⇒ builds JPEG and sends via Gmail SMTP.

- ESP32: subscribed to topic receives message, checks token. If OK, rotates SG90 to 180° for 0.8s and returns to 0°. Serial shows logs confirming receipt and completion.

## 8. Troubleshooting (common problems & fixes)

Problem: ModuleNotFoundError: No module named 'face_recognition'

- Cause: script run outside the face_rec venv or package not installed.

- Fix:

  - Activate venv: source ~/face_rec/bin/activate

  - Install packages: pip install face-recognition opencv-python paho-mqtt

Problem: Encodings file not found: encodings.pickle

- Cause: the script expects the file in its current working directory.

- Fix:

  - Either run the script from the directory containing encodings.pickle:

cd "/home/aman/Desktop/Face Recognition"
python3 facial_recognition_hardware.py

  - Or update ENCODINGS_PATH in script to the absolute path:
 ENCODINGS_PATH = "/home/aman/Desktop/Face Recognition/encodings.pickle"

Problem: ESP32 not receiving HTTP calls (ConnectTimeoutError)

- Cause: phone hotspot often isolates clients (blocks device-to-device traffic). When Pi and ESP are on a hotspot the Pi’s HTTP POST to ESP may time out.

- Fix: Use MQTT over public broker (works because both devices can access internet); or use a home router that allows client-to-client traffic; or set ESP as AP and temporarily connect Pi to it (no internet then).

Problem: Could not open camera index

- Cause: webcam not connected or wrong device index (/dev/video0).

- Fix:

  - Check ls /dev/video*

  - Try other indices: cv2.VideoCapture(1) etc.

  - Ensure no other program is using the camera.

Problem: Gmail send fails

- Cause: wrong app password, lack of 2FA, or network connectivity.

- Fix:

  - Enable 2-step verification on Gmail account and create an App password for SMTP.

  - Use that 16-character app password in script.

  - Consider environment variables instead of plain text in script.

## 9. Security & production notes

- Gmail credentials in script: For testing I used an app password in the script. For production:

  - Use environment variables or a .env file and read os.environ or python-dotenv.

  - Or implement OAuth2 for Gmail (more setup).

- MQTT broker: Using test.mosquitto.org was for testing. For production:

  - Use a secure broker with TLS and authentication (HiveMQ Cloud, CloudMQTT, Mosquitto with TLS).

- Token: the JSON token is a simple shared secret. Consider HMAC or TLS-signed messages if security is important.

- Logs & backups: keep backups of encodings and maintain a log of detection events if needed.

## 10. Commands and quick reference

Activate venv

source ~/face_rec/bin/activate

Run the recognition script

python3 "/home/aman/Desktop/Face Recognition/facial_recognition_hardware.py"

(If running from the directory containing the file:)

cd "/home/aman/Desktop/Face Recognition"
python3 facial_recognition_hardware.py

Stop script (graceful)

- Press CTRL+C in the terminal running the script.

Force stop

pkill -f facial_recognition_hardware.py
# or
killall python3

Check camera device

ls /dev/video*

Test MQTT publish manually (quick test)

source ~/face_rec/bin/activate
python3 - <<'PY'
import json, time, paho.mqtt.client as mqtt
broker="test.mosquitto.org"
topic="face_control/esp32/rotate"
client=mqtt.Client()
client.connect(broker,1883,60)
client.loop_start()
client.publish(topic, json.dumps({"token":"aman_face_access","angle":180}))
time.sleep(1)
client.loop_stop()
client.disconnect()
print("published")
PY

Test HTTP (if ESP used HTTP server)

curl -v -X POST http://<ESP_IP>/rotate -H "Content-Type: application/json" -d '{"token":"aman_face_access"}'

## 11. Example logs and expected output

From Pi terminal when face detected

[INFO] Loaded encodings.
[INFO] Camera opened. System ready.
[ACTION] Authorized face detected: Aman Vajpayee
[MQTT] Published: {'token': 'aman_face_access', 'angle': 180}  rc: 0
[EMAIL] Sent.

From ESP32 Serial Monitor (MQTT subscriber)

[WiFi] Connected! IP: 10.237.37.218
[MQTT] Subscribed to face_control/esp32/rotate
[MQTT] Message arrived on face_control/esp32/rotate
[MQTT] Payload: {"token":"aman_face_access","angle":180}
[MQTT] Token OK - rotating to 180 deg
[MQTT] Rotation complete, returned to 0°

## 12. Next recommended improvements (if you want to evolve system)

- Move Gmail credentials to environment variables (do not keep secrets in source).

- Use a secure MQTT broker (TLS + username/password).

- Add logging & persistence: write detection events to a CSV or SQLite DB with timestamp and filename of the snapshot.

- Add retry/queue for MQTT publishes so commands are not lost if broker is briefly unreachable.

- Add a small web UI to view live detections and history (Flask + small frontend).

- Edge model optimization: replace hog with cnn or use optimized face detection models for better accuracy / performance tradeoffs on Pi.

- Rate-limit or throttle emails and attach low-resolution images to avoid Gmail rate limits.

## 13. Where the final script is saved (for your convenience)

Main script (final, fixed):
 /home/aman/Desktop/Face Recognition/facial_recognition_hardware.py

Backup (original contents before overwrite):
 /home/aman/Desktop/Face Recognition/facial_recognition_hardware.py.bak

(You can open or download the script at that path on your Pi.)

## 14. Quick checklist for handing in / demonstration

- encodings.pickle present and correct (one folder per person test case).

- Pi connected to the same network and venv activated.

- ESP32 uploaded with MQTT subscriber sketch and Serial Monitor open.

- Run the Pi script and demonstrate:

  - Face recognition box + label on the Pi display,

  - Servo rotates on the ESP32,

  - Gmail received with snapshot.