# Project Progress and System Evolution Report

## Introduction

This document describes the complete evolution of my face recognition–based IoT system, starting from a basic facial recognition setup and progressing into a scalable, multi-user, hardware-integrated system capable of performing different physical actions based on the detected individual. The purpose of this document is to clearly explain what changes were made, why they were made, and what the system is now capable of doing.

All explanations are written from my perspective to accurately reflect my development process and learning outcomes.

## Initial System Overview

Initially, my project consisted of a Raspberry Pi performing face recognition using a USB webcam and the face_recognition Python library. The system was able to:

- Capture live video frames from a webcam

- Detect faces in real time

- Match detected faces against a pre-trained dataset

- Identify whether a detected face was known or unknown

At this stage, the system was limited to simple detection and labeling on the screen, without any meaningful hardware interaction or extensibility.

## Introduction of Hardware Control via ESP32

To extend the system beyond visual output, I introduced an ESP32 microcontroller to perform physical actions. The Raspberry Pi and ESP32 communicate using MQTT, which allowed reliable message passing even over a mobile hotspot.

Key improvements made at this stage:

- Implemented MQTT publishing from Raspberry Pi

- Implemented MQTT subscription on ESP32

- Added secure token-based validation in MQTT messages

- Enabled the ESP32 to control a servo motor

This allowed the Raspberry Pi to trigger physical hardware actions remotely upon successful face recognition.

## Transition from Action-Based Logic to Angle-Based Mapping

Initially, hardware control was implemented using action keywords such as servo180 and spin360. While functional, this approach did not scale well beyond a few users.

To address this limitation, I redesigned the logic to use an angle-based mapping system. Instead of assigning actions, each authorized person is now mapped directly to a servo angle.

Example mapping:

- Aman Vajpayee → 0°

- Jake Paul → 20°

- Arjun Kapoor → 40°

- Trump → 180°

This change enabled:

- Simple scaling to many users

- Removal of complex conditional logic

- Cleaner and more maintainable code

## Multi-User Face Recognition Support

The system was upgraded to support ten different individuals, each associated with a unique servo angle. This required:

- Capturing datasets for each individual

- Retraining the facial encodings using model_training.py

- Ensuring exact name matching between dataset folders and code logic

This resulted in a robust multi-user recognition pipeline capable of performing different physical responses based on who is detected.

## Cooldown Mechanism Implementation

To prevent repeated triggering of hardware and email alerts when a person remains in front of the camera, I implemented a per-user cooldown system.

Key aspects:

- Each recognized person has an independent cooldown timer

- Hardware actions and alerts are only triggered once per cooldown period

- Cooldown duration is configurable

This significantly improved system stability and prevented unnecessary hardware wear and email spam.

## Email Alert Integration

The system was further enhanced by integrating Gmail alerts. Whenever an authorized face is detected:

- A snapshot of the frame is captured

- The image is encoded as JPEG

- An email is sent with the image attached

- The email includes the detected person’s name and timestamp

This feature allows real-time remote monitoring and provides visual confirmation of detections.

## Camera and Stability Fixes

During development, I encountered multiple camera-related issues such as frame timeouts and device freezes. These were resolved by:

- Verifying correct video device selection

- Testing camera access using OpenCV directly

- Replugging and reinitializing the USB webcam

- Ensuring stable power and correct resolution settings

These steps resulted in a reliable real-time video feed suitable for continuous operation.

## Final System Architecture

The final system operates as follows:

- The Raspberry Pi captures frames from the webcam

- Faces are detected and encoded in real time

- Detected faces are matched against trained encodings

- If a recognized face is found:

  - The corresponding servo angle is retrieved

  - An MQTT message is sent to the ESP32

  - The ESP32 rotates the servo to the specified angle

  - An email alert with snapshot is sent

- A cooldown timer prevents immediate retriggering

This architecture cleanly separates:

- Decision-making (Raspberry Pi)

- Hardware execution (ESP32)

## System Capabilities After All Modifications

After all changes, the system is now capable of:

- Recognizing up to ten different individuals

- Performing different hardware actions per individual

- Remotely controlling hardware via MQTT

- Sending automated email alerts with visual proof

- Running continuously with stability safeguards

- Scaling easily to additional users or actions

## Learning Outcomes

Through this development process, I gained hands-on experience with:

- Computer vision and face recognition

- IoT communication using MQTT

- Embedded system integration

- Multithreading in Python

- System debugging and error resolution

- Designing scalable and maintainable logic

## Conclusion

This project evolved from a basic face recognition demo into a complete IoT-enabled intelligent system. By progressively improving the design and addressing real-world issues such as scalability, reliability, and hardware control, I was able to build a robust and extensible platform suitable for academic evaluation and future research work.

This document represents the complete progression and final state of my system.

## How My System Follows the Complete IoT Cycle

An Internet of Things (IoT) system is considered complete when it follows a full cycle consisting of sensing, data acquisition, processing, communication, actuation, feedback, and monitoring. My system successfully implements every stage of this cycle in a real-world, working setup.

## 1. Sensing (Data Generation Layer)

The IoT cycle begins with sensing the physical environment.

In my system:

- A USB webcam connected to the Raspberry Pi acts as the primary sensor.

- The webcam continuously captures live video frames of the surrounding environment.

- These frames contain raw visual data representing human presence.

This step converts physical-world information (faces) into digital data, which is the first and most critical stage of any IoT system.

## 2. Data Acquisition and Preprocessing

Once data is sensed, it must be acquired and prepared for processing.

In my system:

- Each video frame is captured using OpenCV.

- Frames are converted from BGR to RGB format.

- Face locations are extracted using the HOG-based face detector.

- Facial features are encoded into numerical vectors.

At this stage:

- Raw image data is transformed into structured, machine-readable information.

- Noise and irrelevant data are reduced.

This step ensures the data is suitable for intelligent decision-making.

## 3. Edge Processing and Decision Making (Edge Intelligence)

This stage represents the intelligence of the IoT system.

In my system:

- Facial encodings are compared with pre-trained encodings stored locally.

- The system determines whether a detected face is:

  - Known (authorized)

  - Unknown (unauthorized)

- If authorized, the system identifies exactly which person is detected.

- A decision is made locally on the Raspberry Pi without relying on cloud computation.

This qualifies as Edge AI, where computation happens close to the data source, reducing latency and increasing reliability.

## 4. Data Mapping and Rule-Based Logic

After recognition, the system applies predefined logic.

In my system:

- Each authorized person is mapped to a specific servo angle.

- This mapping is stored in a dictionary-based configuration.

- The system dynamically selects the correct angle based on identity.

This stage converts identity information into actionable commands, making the system scalable and deterministic.

## 5. Communication Layer (Network & Protocols)

Once a decision is made, it must be communicated to another device.

In my system:

- The Raspberry Pi publishes messages using the MQTT protocol.

- Messages are sent to a public MQTT broker.

- The payload is formatted as structured JSON data.

- A secure token is included for validation.

This step demonstrates:

- Lightweight IoT communication

- Decoupling of devices

- Reliable message delivery across networks

## 6. Actuation (Physical Action Layer)

This stage represents the physical response of the IoT system.

In my system:

- An ESP32 subscribes to the MQTT topic.

- Upon receiving a valid message:

  - The payload is verified

  - The servo motor rotates to the specified angle

- Each detected individual triggers a unique physical motion.

This converts digital decisions back into physical-world actions, completing the cyber-physical loop.

## 7. Feedback and Notification

A complete IoT system provides feedback or acknowledgment.

In my system:

- A snapshot is captured at the moment of detection.

- An automated Gmail alert is sent with:

  - Detected person’s name

  - Timestamp

  - Image attachment

- Serial logs on the ESP32 confirm command execution.

This feedback ensures transparency, traceability, and system validation.

## 8. Monitoring and Control Safeguards

Continuous operation requires safeguards and monitoring.

In my system:

- A cooldown mechanism prevents repeated triggers.

- Each person has an independent cooldown timer.

- Camera availability is constantly checked.

- MQTT reconnection logic ensures resilience.

These mechanisms ensure long-term stability and safe operation.

## 9. Continuous Loop (Closed IoT Cycle)

After actuation and feedback:

- The system immediately resumes sensing.

- The entire process repeats continuously.

- No manual intervention is required.

This establishes a closed-loop IoT system, which is a defining characteristic of a complete IoT implementation.

## Summary of IoT Cycle Mapping

| IoT Stage | Implementation in My System |
| --- | --- |
| Sensing | USB webcam captures live video |
| Data Acquisition | Frame capture and preprocessing |
| Edge Processing | Face recognition on Raspberry Pi |
| Decision Logic | Identity-to-angle mapping |
| Communication | MQTT with JSON payloads |
| Actuation | ESP32-controlled servo motor |
| Feedback | Gmail alerts and serial logs |
| Monitoring | Cooldown and fault handling |
| Loop Closure | Continuous real-time operation |

## Conclusion

My system follows the complete IoT cycle from sensing to actuation and feedback using real hardware, real communication protocols, and intelligent edge processing. It demonstrates a practical, scalable, and production-aligned IoT architecture that integrates computer vision, embedded systems, and network communication into a single closed-loop system.

This makes the project not just a face recognition application, but a full-fledged IoT system suitable for academic evaluation and real-world deployment.